Linux Networking

IP classes:

- Class A – the classful subnet mask is /8. The value starts from 1-126

- Class B – the classful subnet mask is /16.The value starts from 128-191.

- Class C – the classful subnet mask is /24.The value starts from 192-223

Now we should create an new network adapter

select VM ---> settings ---> Add Network adapter

# ifconfig (check the IF name)

# nmcli device show

# nmcli device connect <network adapter name>

set the static IP address

selete VM ---> settings ---> Add Network adapter

# ifconfig (check the IP name)

# nmcli device show

# nmcli connection

Now again we should want to create a new network addapter

# nmcli con add type ethernet con-name ens160 ifname ens160 ipv4.addresses 192.168.159.51/24 ipv4.gateway 192.168.0.1 autoconnect yes ipv4.method manual

command to check gateway:

# netstat -rn

# route -n

TO Modify Connection

First we should down the system

- nmcli con mod ens37 ipv4.addresses 192.168.2.100/24

- nmcli con mod ens37 ipv4.gateway 192.168.2.200

nmcli con mod ens36 ipv4.dns 8.8.8.8

- nmcli con mod ens36 ipv4.method manual

- nmcli con mod ens36 connection.autoconnect yes

To change all details at same time:

- nmcli con modify ens160 ipv4.addresses 192.168.2.100/24 ipv4.gateway 192.168.2.1 ipv4.method manual connection.autoconnect yes